# Whats

This solution goal is alerting the incoming of a flash flood therefore a satisfying solution is needed to serve DID (Department of Irrigation and Drainage) and its responders who will monitor the drainage canals. Based on the root of the problem, there are several user requirements that the solution need to satisfy.

## Accurate water level data & remote monitoring

The DID’s responders wont trust or act based on readings that could be false and they also can’t come to every site in person to do checking. Due to that, the system would require to be both precise sensor data and can be remotely accessed so that the user side can view the conditions without needed to there physically. A system already has been developed by Yahya and Homam (2025) that can support this, their system used an ultrasonic sensor with the usage of LoRa transmission to an IoT platform, letting water level data be viewed remotely at real time (Yahya & Homam, 2025).

## Timely warning & alerts reach

Flooding in drainage canals can rise quickly during heavy rain, so the system needs to detect and send out warnings fast enough for DID to respond before the situation becomes dangerous. A warning that arrives too late defeats the whole purpose of an "early" warning system. That is why the system will use SMS rather than a manual checkup because SMS specifically is one of the fastest, most reliable way to notify to corresponding individuals and able to alert the at-risk residents once water crosses a threshold (Azid et al., 2015). As extra layer of alert system, the system would also come with a siren that’s also responsible to notify the nearby locals.

## Resilient on storms/outages

Outages are most likely happening when a flood is happening, so a system that only works on normal condition would fails at the moment when its needed most. For the target customer, this failure is not what they are looking for because they need the system to work during emergencies and can alerts the corresponding responders. Therefore, a system the system would need a backup power such as emergency batteries or a backup solar power storage.

## Low maintenance

Because the sensors are installed in drainage canals which are a harsh, hard-to-access environment, the system should not require frequent manual servicing or recalibration. If the system needs constant attention to keep working, it becomes a burden rather than a solution. DID has limited staff and resources, so the solution needs to run reliably with minimal upkeep.

## Flood detection

Beyond just reporting water levels, the system should be able to recognize when conditions indicate an actual flood risk and not just report a number. Without this interpretation step, responders have to manually judge and read every reading themselves which bring back manual monitoring system that this solution is meant to solve.

# Hows

## Sensor Accuracy

Every other requirement depends on the raw water level reading being correct first. The reading needs to be accurate, timely, and one that can tell a genuine flood apart from normal fluctuations. This is the foundational requirement behind “accurate water level data”.

## Detection-to-Alert Latency

The time between the sensor first reading a dangerous threshold and the DID being acknowledged is what timely warning comes down to in practice, since a flash flood can outrun a slow alert pipeline.

## Fault Alert Latency

Distinct from the public flood alert: how quickly a hardware or communication fault at the field station is reported to maintenance staff. This directly serves “low maintenance”, catching problems before they silently degrade the water-level data DID is relying on.

## Alert Channel Redundancy

This includes the number of independent paths an alert can travel over, so that a single failed channel during a storm or any other crisis does not stop the warning from reaching the responders and residents. In this case, SMS, siren, and cellular push. This serves the “alerts reach” need.

## Enclosure IP rating

This serves as the durability spec for the “resilient in storms/outages” and “low maintenance”, keeping field electronics functioning through the wet, harsh canal environment without frequent servicing.

## Remote Monitoring

Lets DID staff check canal conditions and system health from a central dashboard instead of visiting each site. This serves "remote monitoring" directly and "low maintenance" indirectly.

## Rate of Rise

A derived trigger (Δwater-level / Δtime), distinct from the absolute level itself. This is the functional requirement that most directly operationalizes "flood detection" a fast rate of rise is the signature of a flash flood, as opposed to normal tidal or rainfall-driven fluctuation.

# Competitive Assessment

Brief Introduction of Competitor 1

HDLS is a local Malaysian company that once partnered with the Malaysian Government to create a flood monitoring system for Northern Peninsular Malaysia. HDLS used AQUAS flood monitoring products alongside its own satellite communication and remote-control services to piece together a solution for the government. (Innovative Flood Sensor Monitoring Deal Malaysia, 2025) Therefore, to give a clearer picture of the competitors, AQUAS is a Taiwan-based company that specializes in all sorts of water quality and flood monitoring systems and has decades of history and multinational customers across the globe, including Malaysia. (Aquas Inc, n.d.)

## Competitor Specification  (Aquas Inc, n.d.)

| Solutions | Sensor Accuracy | Detection to Alert Latency | Fault-alert latency | Alert channel redundancy |
| --- | --- | --- | --- | --- |
| Ours | +-5 mm | <5s | <60s | 3 |
| HDLS+AQUAS | +-1mm | 15s | 15s | 5 |

| Solutions | Enclosure IP rating | Rainfall + Level Fusion | Remote Monitoring | Rate of Rise Detection |
| --- | --- | --- | --- | --- |
| Ours | IP65 | 1-3 h | 5 years | >10 |
| HDLS+AQUAS | IP68 | 1-3 h | 10 years | Not applicable |

Although the solutions are from HDLS, the equipment and systems are provided by AQUAS; therefore, the specs above are from AQUAS ' website. The detection-to-alert latency and fault-alert latency are based on the response time of the system. It can support up to 5 alert channels via Ethernet, Bluetooth, cellular, satellite, Wi-Fi, through PLC, DCS, HMI communication or HART protocol. It is IP68, meaning it is dust-tight and can withstand submergence up to an hour. Our solution won only on faster detection alert latency. However, HDLS solutions focus on river and sewage monitoring detection, while our solution focuses more on flash floods caused by rainfall. Our solution gains a slight advantage by being able to detect rainfall information that HDLS couldn’t.

## Brief Introduction of Competitor 2

SureChem is also a local Malaysian company that was founded 40 years ago. They specialized in monitoring and measuring data that is related to environmental fields. It is currently offering a solution similar to ours to detect and monitor flash floods.(Home | Surechem, n.d.)

## Competitor Specification (Observator Instrument, n.d.) (Seba Hydrometrie., n.d.)

| Solutions | Sensor Accuracy | Detection-to-alert latency | Fault-alert latency | Alert channel redundancy |
| --- | --- | --- | --- | --- |
| Ours | +-5 mm | <5s | <60s | 3 |
| SureChem | +-2-11.4mm, 2mm | <2min | <2min | 1 |

| Solutions | Enclosure IP rating | Rainfall + Level Fusion | Remote Monitoring | Rate of Rise Detection |
| --- | --- | --- | --- | --- |
| Ours | IP65 | 1-3 h | 5 years | >100mm/h |
| SureChem | IP65, IP68 | - | 25 years | 1% accuracy at 200mm/h, 3% accuracy at 380mm/h |

The data above are extracted from SureChem equipment, as their solution can be different depending on the buyer's requirements. In short, they offer 3 products in their solution that can be used together or separately.  Below is the analysis of their specifications.

Their rain gauge has an accuracy of 2- 11.4 mm, which is calculated based on the rate-of-rise detection setting, while their radar sensor for detecting water level has an overall accuracy of 2 mm. Our solution has lower accuracy but has a higher rate-of-rise detection.

Their pressure sensors have a 2-minute measuring interval, as the weakest link in the chain, meaning the interval time to calculate and alert will take at least 2 minutes. Furthermore, their alert channel only supports SMS up to 8 recipients.

The pressure sensors have an IP65 rating, while the radar sensors have an IP68 rating. As their solution doesn’t depend on rainfall data like ours, they don’t have a rainfall +level fusion rating. This might be an advantage for them, as it is a less dependent solution.

# References

Aquas Inc. (n.d.). Made in Taiwan, World Leader in the Market. Aquas Inc. Retrieved 	September 23, 2026, from https://www.aquas.com.tw/en/a1/About-AQUAS.html

Aquas Inc. (n.d.). Wastewater Complete Solution. Aquas Inc. Retrieved September 23, 2026, 	from https://www.aquas.com.tw/en/a4-1175-5385/Sewer-Network-Flow-and-Water-	Quality-Monitoring-System.html

Azid, S. I., Sharma, B. N., Raghuwaiya, K., Chand, A., Prasad, S., & Jacquier, A. (2015). SMS based flood monitoring and early warning system. ARPN Journal of Engineering and Applied Sciences, 10(15), 6387–6391.

Home | Surechem. (n.d.). Www.Surechem.Com.My. Retrieved September 23, 2026, from 	https://www.surechem.com.my/

Innovative Flood Sensor Monitoring Deal Malaysia. (2025, March 24). AST Networks. 	https://ast-networks.com/insights/news/flood-sensor-monitoring/

Observator Instrument. (n.d.). RIM7499 rain gauge product specifications [Product 	specification]. 	https://www.surechem.com.my/sites/default/files/node/product/document/20	22-03/doc_2033_181004023731.pdf

Seba Hydrometrie. (n.d.). Pressure sensor pneumatic gauge PS-Light-2 [Product 	specification]. 	https://www.surechem.com.my/sites/default/files/node/product/document/20	22-03/D03_Pressure_sensor_pneumatic_gauge_PS-Light-2_e.pdf

Seba Hydrometrie. (n.d.). Radar Sensor Type SEBAPuls 8/15/30/20/120[Product 	specification]. 	https://www.surechem.com.my/sites/default/files/node/product/document/2022-	03/B23_Radar_Sensor_Type_SEBAPULS_e.pdf

Yahya, N. L., & Homam, M. J. (2025). Real-time flood monitoring alert system with LoRa and IoT. Evolution in Electrical and Electronic Engineering, 6(1), 90–102. https://doi.org/10.30880/eeee.2025.06.01.013